SAFETY DATA SHEET

OSHA HCS (29 CFR 1910.1200)

# SECTION 1: PRODUCT AND COMPANY IDENTIFICATION

| Product identifier |  |  |
| --- | --- | --- |
|  | Chemical Family / Chemical Name | Mixture |
|  | CAS No. | Mixture |
|  | Trade Name | MC-1840 |
|  | Product Code | None |
| Relevant identified uses of the substance or mixture and uses advised against |  |  |
|  | Identified Use(s) | Precipitant |
|  | Uses Advised Against | None |
|  | Company Identification | Meitler Consulting, Inc 16979 Chieftain Rd PO Box 444 Tonganoxie, KS 66086 USA |
|  | Telephone | (913) 422-9339 |
|  | Fax | (913) 422-9344 |
|  | E-mail |  |
| Emergency telephone number |  |  |
|  | Emergency Phone No. | Transportation Emergency: CHEMTREC 24 hr. 1-800-424-9300 / 1 (703) 527-3887 (Collect calls accepted) |

# SECTION 2: HAZARDS IDENTIFICATION

| Classification of the substance or mixture |  |  |
| --- | --- | --- |
|  | OSHA HCS (29 CFR 1910.1200) | Skin Irrit. 1B; Eye Dam. 1; Acute Toxicity (Oral = 3/ Inhalation = 4) |
| Label elements |  |  |
|  | Hazard Symbol |  |
|  | Signal Word(s) | DANGER |
|  | Hazard Statement(s) | Causes serious eye damage .Causes severe skin burns and eye damage . May be corrosive to metals . Very toxic to aquatic life Toxic if swallowed. Harmful if inhaled |
|  | Precautionary Statement(s) | Wear protective gloves/protective clothing/eye protection/face protection. Wash thoroughly after handling. IF ON SKIN: Wash with plenty of soap and water. If irritation (redness, rash, blistering) develops, get medical attention. IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Get immediate medical attention. IF SWALLOWED: rinse mouth. Do NOT induce vomiting. Seek medical treatment. |
| Other hazards |  | None |
| Additional Information |  | Warning - this preparation contains a substance not yet tested completely. |

# SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS

| ### Hazardous Ingredient(s) | ### % wt. | ### CAS No. |
| --- | --- | --- |
| Sodium Hydrosulfide Solution Water | Trade Secret Trade Secret | 16721-80-5 7732-18-5 |
| Additional Information - Substances in the product which may present a health or environmental hazard, or which have been assigned occupational exposure limits, are detailed below.: |  |  |

# SECTION 4: FIRST AID MEASURES

| Description of first aid measures |  |  |  |
| --- | --- | --- | --- |
|  | Inhalation |  | Call a Poison Center or doctor/physician if exposed or you feel unwell. Remove person from source of exposure to fresh air. If breathing is difficult, give oxygen. If not breathing, give artificial respiration using a pocket mask or resuscitation device. Direct mouth contact should be avoided if possible due to the potential for residual corrosive liquid around the person's mouth and airways. |
|  | Skin Contact |  | Immediately remove contaminated clothing and shoes. In case of contact, immediately flush skin with soap and plenty of water. Get medical attention immediately if irritation (redness, rash, blistering) develops and persists. |
|  | Eye Contact |  | In case of contact, immediately flush eyes with plenty of water for at least 20 minutes. Get medical attention immediately. Person may be kept in a dark room with ice compresses applied to eyes and forehead until medical treatment is available. Speed in treatment may prevent permanent eye damage. |
|  | Ingestion |  | Call a physician or a poison control center immediately. If vomiting occurs, keep head low so that stomach contents do not enter the lungs. If conscious, rinse the mouth out several times with cold water and spit out. Give one or two cups of water or milk. This may be followed by gastric antacids, such as milk of magnesia or aluminum hydroxide. Stop if victim becomes nauseated. DO NOT INDUCE VOMITING unless instructed to do so by medical personnel. Never give anything by mouth to an unconscious person. If the victim stops breathing: administer artificial respiration with the aid of a pocket mask equipped with a one-way valve or other proper respiratory medical device. |
| Most important symptoms and effects, both acute and delayed |  |  | Some medical protocols prescribe the use of amyl nitrite as part of first aid treatment. Do not use amyl nitrite treatment if oxygen is not available. Amyl nitrite is only a useful adjunct during the first 10 to 15 minutes following exposure. Once breathing is restored, provide a high flow of oxygen and amyl nitrite if appropriate Symptoms of pulmonary edema may be delayed for 24 to 72 hours after initial exposure. Therefore, hospitalization and medical observation is advisable during this period. |
| Indication of any immediate medical attention and special treatment needed |  |  | Treat symptomatically |

# SECTION 5: FIRE-FIGHTING MEASURES

| Extinguishing Media |  |  |  |
| --- | --- | --- | --- |
| -Suitable Extinguishing Media |  | Non-combustible / Non-flammable. As appropriate for surrounding fire. Lower Explosive Limit: 4% (hydrogen sulfide) Upper Explosive Limit: 46% (hydrogen sulfide) |  |
| -Unsuitable Extinguishing Media |  | As appropriate for surrounding fire. |  |
| Special hazards arising from the substance or mixture |  | Product solution is non-flammable. However, trace levels of flammable hydrogen sulfide gas are continuously released in air, especially when product is heated or exposed to acids. Gas may form explosive mixtures in air. Do not cut open or apply heat sources to containers. Thermal decomposition ("burning") may evolve toxic and irritating combustion by-products - hydrogen sulfide. |  |
| Advice for fire-fighters |  | Avoid breathing vapors, gases and fumes. Do not touch, handle or walk-through spilled liquid. Firefighters should wear a positive pressure-demand self-contained breathing apparatus (SCBA) and full protective gear. Containers may build up pressure if exposed to radiant heat. Water can be used to cool and protect exposed material. Do not allow runoff to enter sewers or waterways. Move containers away from fire area if safe to do so. |  |

# SECTION 6: ACCIDENTAL RELEASE MEASURES

| Personal precautions, protective equipment and emergency procedures | Put on protective equipment before entering danger area. Wear protective gloves/protective clothing/eye protection/face protection. |
| --- | --- |
| Environmental precautions | Prevent substance entering sewers. |
| Methods and material for containment and cleaning up | Cover spills with inert absorbent material. Transfer to a container for disposal or recovery. |
| Reference to other sections | None |
| Additional Information | None |

# SECTION 7: HANDLING AND STORAGE

| Precautions for safe handling | Wear protective gloves/protective clothing/eye protection/face protection. Avoid breathing vapour, gas or mist. Use only with adequate ventilation. |  |
| --- | --- | --- |
| Conditions for safe storage, including any incompatibilities |  |  |
| -Storage temperature | Keep away from heat or flame. Store in a cool, dry, well-ventilated area out of direct sunlight (<104 deg F, 40 deg C). Keep container tightly closed. Keep away from children. |  |
| -Incompatible materials | Not compatible with copper, zinc, aluminum or their alloys (e.g., brass, bronze, galvanized metals, etc.). Corrosive to steel above 150° F (65.5° C). |  |
| Specific end use(s) | Precipitant |  |

# SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

| Occupational Exposure Limits |  |  |  |  |
| --- | --- | --- | --- | --- |
| Recommended monitoring method |  | Real-time (electrochemical sensors) |  |  |
| Exposure controls |  |  |  |  |
| ### Appropriate engineering controls |  |  |  | ### Use outdoors or indoors only with adequate general and local exhaust ventilation. Maintain exposures to hydrogen sulfide below occupational exposure limits. The use of hydrogen sulfide air monitoring detectors with alarms is recommended for poorly ventilated areas and confined spaces. |
| Personal protection equipment |  |  |  |  |
|  | Eye/face protection |  |  | At a minimum, chemical splash goggles or face shield over safety glasses or goggles should be worn at all times when handling. A full face piece should be worn with SCBA or air-line respirator. |
|  | Skin protection (Hand protection/ Other) |  |  | The use of chemical-resistant gloves made of neoprene rubber are recommended as minimum industrial skin protection when handling product or performing spill clean-up. Chemical resistant apron, and/or suit and boots should be worn to prevent skin contact. Chemical protective clothing constructed of DuPont Tychem Responder or equivalent material may be used for spill clean-up. |
|  | Respiratory protection |  |  | Engineering controls should be implemented preferentially to reduce exposures. If working near open container, storage vessel opening or open tank truck dome cover, wear self-contained breathing apparatus, or positive pressure demand air-line respirator if there is a potential for exposure. Air-purifying (cartridge) respirators should not be used, except for escape purposes, due to the possible presence of hydrogen sulfide. |
|  | Thermal hazards |  |  | Not normally required. Use gloves with insulation for thermal protection, when needed. |
| ### Environmental Exposure Controls |  |  |  | Collect all precipitate. Disposal should be in accordance with local, state or national legislation. |

# SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES

| Information on basic physical and chemical properties |  |
| --- | --- |
| Appearance | Liquid |
| Color | Dark Amber to Green |
| Odor | Slight Sulfur |
| Odor Threshold (ppm) | Not available |
| pH | 12.0 - 12.7 |
| Boiling point/boiling range (°C): | >212 °F |
| Flash Point (°C) | Non-combustible / Non-flammable |
| Evaporation Rate | 1.0 |
| Flammability (solid, gas) | Not applicable |
| Explosive Limit Ranges | Not applicable |
| Vapor pressure (Pascal) | N/A |
| Vapor Density (Air=1) | Not available |
| Specific Gravity | 1.080 - 1.110 |
| Solubility (Water) | Complete |
| Solubility (Other) | Not available |
| Partition Coefficient (n-Octanol/water) | Not available |
| Auto Ignition Point (°C) | Non-combustible / Non-flammable as supplied |
| Decomposition Temperature (°C) | Not available |
| Kinematic Viscosity | Not available |
| Explosive properties | Not explosive |
| Oxidizing properties | Not oxidizing |
| Other information | Not available |

# SECTION 10: STABILITY AND REACTIVITY

| Reactivity | Stable under normal conditions |
| --- | --- |
| Chemical stability | Stable. |
| ### Possibility of hazardous polymerization | Will not occur |
| Conditions to avoid | Incompatible materials |
| Incompatible materials | Avoid contact with strong oxidizing agents and acids. Sodium hydrosulfide solutions attack zinc, aluminum, copper and alloys of these metals. |
| Hazardous decomposition product(s) | Contacts with acids may liberate flammable and poisonous hydrogen sulfide gas. . |

# SECTION 11: TOXICOLOGICAL INFORMATION

Exposure routes: Skin Contact, Eye Contact

| Acute toxicity | Oral: No information available Dermal: No information available |
| --- | --- |
| Skin Irritation/Corrosivity | No information available |
| Sensitization | No information available |
| Repeated dose toxicity | No information available |
| Carcinogenicity | No information available |
| NTP; IARC; ACGIH; OSHA; NIOSH. No.; No.; No.; No.; No.. |
| Mutagenicity | No information available |
| Toxicity for reproduction | No information available |

# SECTION 12: ECOLOGICAL INFORMATION

| ### Toxicity |  |
| --- | --- |
| ### Short term | Not available |
| ### Long Term | Not available |
| ### Persistence and degradability | Not available |
| ### Bioaccumulative potential | Not available |
| ### Mobility in soil | Not available |
| ### Results of PBT and vPvB assessment | Not classified as PBT or vPvB. |
| ### Other adverse effects | None known. |

# SECTION 13: DISPOSAL CONSIDERATIONS

| ### Waste treatment methods | llegislation. Consult |
| --- | --- |

# SECTION 14: TRANSPORT INFORMATION

|  | Land transport (U.S. DOT) | Sea transport (IMDG) | Air transport (ICAO/IATA) |
| --- | --- | --- | --- |
| UN number | UN 3267 | UN 3267 | UN 3267 |
| Proper Shipping Name | Corrosive liquid, basic, organic, n.o.s. (Sodium Hydrosulfide Solution) |  |  |
| Transport hazard class(es) | 8 (6.1) | 8 (6.1) | 8 (6.1) |
| Packing group | III | III | III |
| Hazard label(s) | Corrosive, Toxic | Corrosive, Toxic | Corrosive, Toxic |
| Environmental hazards | Yes | Yes | Yes |
| Special precautions for user | DO NOT STORE NEAR ACID |  |  |
| Transport in bulk according to Annex II of MARPOL73/78 and the IBC Code: Not established. |  |  |  |

# SECTION 15: REGULATORY INFORMATION

| Safety, health and environmental regulations/legislation specific for the substance or mixture: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TSCA (Toxic Substance Control Act) - Inventory Status: All components listed or polymer exempt. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Designated Hazardous Substances and Reportable Quantities (40 CFR 302.4): |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Chemical Name | CAS No. |  |  | Typical %wt. |  |  |  | RQ (Pounds) |  |  |
|  |  |  | - | - |  |  | - |  |  |  | - |  |  |
| SARA 311/312 - Hazard Categories: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fire Sudden Release Reactivity Immediate (acute) Chronic (delayed) |  |  |  |  |  |  |  |  |  |  |  |  |
| SARA 313 - Toxic Chemicals (40 CFR 372): |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Chemical Name |  |  |  |  | CAS No. |  |  |  |  | Typical %wt. |
|  |  |  | None |  |  |  |  | ---- |  |  |  |  | ---- |
| SARA 302 - Extremely Hazardous Substances(40 CFR 355): |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Chemical Name |  |  | CAS No. |  |  |  | Typical %wt. |  | TPQ (pounds) |  |
|  |  |  | None |  |  | ---- |  |  |  | ---- |  | ---- |  |
| California Proposition 65 List: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Chemical Name |  |  | CAS No. |  |  |  | Type of Toxicity |  |  |  |  |
|  |  | None |  |  | ---- |  |  |  | ---- |  |  |  |  |

# SECTION 16: OTHER INFORMATION

- HMIS RATINGS:  Health=2,  Flammability=1,  Reactivity=2

- HMIS HAZARD INDEX: 0=MINIMAL, 1=SLIGHT, 2=MODERATE, 3=SERIOUS, 4=SEVERE

| The following sections contain revisions or new statements: |  |  |
| --- | --- | --- |
| Date of preparation: January 17, 2021 |  |  |
| Additional Information: None |  |  |

Disclaimer: We believe the statements, technical information and recommendations contained herein are reliable, but they are given without warranty or guarantee of any kind. The information contained in this document applies to this specific material as supplied. It may not be valid for this material if it is used in combination with any other materials. It is the user’s responsibility to satisfy oneself as to the suitability and completeness of this information for the user’s own particular use.